# Raw Dataset Info

To create the model, a raw data set was generated which contains 500 records of ground motion records in the format used in NGA-West2 Database.

The first sheet consists of this data.

RSN - Record Sequence Number — unique integer ID assigned to each ground-motion record in the NGA-West2 database. Primary key linking metadata, targets, and raw waveforms across all sheets. Used only for traceability.

Magnitude - This is used to characterise the seismic source. Moment magnitude Mw which is the logarithmic measure of total seismic energy released by the earthquake. Higher Mw → longer shaking duration and richer low-frequency content. This feature is used in the regression model (log-domain correlation with WAL). Governs the amplitude of ground shaking that emanates from the shake table. It is dimensionless.

Vs30(m/s) - Time-averaged shear-wave velocity in the top 30 m of soil. Soft soils amplify long-period energy. Controls frequency-dependent amplification of the signal before it arrives at the shake table foundation. Used as log(Vs30) feature in regression.

Epicentral_Distance (km) - Horizontal distance from the earthquake epicentre to the recording station. Used as log(distance+1) feature to capture attenuation effects on PGA.

Depth(km) - Depth of the earthquake under epicenter. Shallow events (<20 km) produce more impulsive, high-frequency records; deep events have longer travel paths. Affects the frequency content and duration of the record. Not a direct regression feature.

Duration_original(s) - Total recorded duration of the raw ground-motion file BEFORE any standardisation. Records shorter than 60 s will be zero-padded; longer records are cropped around the PGA peak to a 60 s window as per pre-processing methodology of paper. The paper requires all inputs to the 1D-CNN to be exactly 3 000 time steps (60 s × 50 Hz). This column tells how much padding or cropping is needed.

Sampling_Freq (Hz) - Original sampling frequency of the accelerogram. This is to be standardised to 50Hz using polyphase resampling.

PGA_X (g) - Peak Ground Acceleration in the X (longitudinal) direction, in units of g (1 g ≈ 9.807 m/s²). This is the raw, un-normalised scalar value of the strongest acceleration in the horizontal X channel. Primary regression feature. PGA_X directly controls the energy input to the structure and is the strongest predictor of WAL at all 9 measuring points.

PGA_Y - Peak Ground Acceleration in the Y (transverse) direction, raw scalar. Secondary regression feature. The Y channel is normalised so that its peak equals 0.85 × the X-channel peak. This simulates the bi-directional shake table excitation where the transverse component is 85 % of the primary direction.

PGA_Z - Peak Ground Acceleration in the Z (vertical) direction, raw scalar. Tertiary regression feature. The vertical channel is normalised to 0.65 × X-peak. Vertical excitation mainly excites axial modes and is relevant for floor-level vibration comfort measurements (EI building).

Dominant_Freq_X (Hz) - Dominant (peak spectral) frequency of the X-direction accelerogram. Computed as the frequency at the peak of the Fourier amplitude spectrum. If the dominant frequency of the input matches the natural frequency of a building story, resonance occurs and WAL spikes. Important for interpreting which measuring points exceed thresholds.

Dominant_Freq_Y - Cross-directional resonance can increase torsional strains.

Dominant_Freq_Z - Relevant for vertical vibration comfort Estimation.

# Pre-processing of the dataset

from scipy.signal import resample_poly: For polyphase sampling we import the following from scipy.signals library. This function resamples signals at a different frequency without introducing aliasing for higher original sampling frequencies

Anti-aliasing Filter first uses a low pass to remove frequencies higher than the new sampling frequency/2 to prevent aliasing and then down samples it by removing the excess samples at equal intervals.

Polyphase filtering makes the process faster by splitting the filter coefficients into M parallel branches that are evaluated at the same time with proper phase difference that is added together to make the filter again.

from math import gcd: To find the gcd for the downsampling process we use gcd function of math library

from sklearn.model_selection import train_test_split: For true shuffling and splitting of the data into learning and test datasets we use the above library.

We do the pre-processing for the ground motion data records and their raw acceleration Time series:-

1. Normalisation and Scaling of Data

2. Resampling at 50Hz

3. Duration Standardisation acc to given convention

4. WAL Calculation

The Simplified waveform used to generate the synthetic data of the earthquake time series: decayed sinusoid + noise

Creates a slowly-decaying amplitude envelope.

np.exp(-0.08*t) makes the signal die away over time.

np.sin(2π×0.15×t) creates a slow modulation that rises and falls and clips negatives so the envelope is always ≥ 0.

A real earthquake accelerogram has three physical phases:

silence → build-up → strong motion → decay.

The envelope simulates this shape

envelope = np.exp(-0.08 * t) * np.maximum(0, np.sin(2*np.pi*0.15*t))

Multiplication by 980.665 converts the PGA from g into gal (cm/s²). This ensures the peak of the synthesised signal matches the PGA value from the dataset. Since our preprocessing only occurs on long time-series data we simulate that by creating this signal from the envelope designed before and a carrier wave which is the actual oscillation: a sinusoid at a characteristic frequency. Noise is also added (instrument noise and small background vibrations

# DTHA

After the pre-processing of the raw dataset to make a normalised data set, this is used to carry out DTHA (Dynamic Time-History Analysis) on the SSS (Shake Table- Soil - Surrounding Building System) to get the relevant WAL (Weighted Acceleration Level) for each of the records at the 9 measuring points on the three buildings in the SSS. These are connected by the primary key of the RSN.

The WAL is our target variable in the Supervised learning we will be carrying out to help predict the vibration comfort of the building prior to a shake table test.

Here we have created dummy values of the WAL since DTHA is not possible.